# დანართი № 3.	რუკაზე გამოყენებული ფერები

CHART SYMBOLS

| Culture, except highways and roads; outlines of large cities, grids and graticules; spot elevations; danger lines and off-shore rocks; names and lettering except for aeronautical and hydrographic features |  | BLACK |
| --- | --- | --- |
| Built-up areas of cities |  | BLACK Stipple |
| Highways and roads | Optional colours | BLACK Half-tone RED |
| Built-up areas for cities (alternative to black stipple) |  | YELLOW |
| Contours and topographic features: Items 1 through 10 of Appendix 2 Hydrographic features: Items 39 through 41 of Appendix 2 |  | BROWN |
| Shore lines, drainage, rivers, lakes, bathymetric contours and other hydrographic features including their names or description |  | BLUE |
| Open water areas |  | BLUE Half-tone |
| Salt lakes and salt pans |  | BLUE Stipple |
| Large non-perennial rivers and non-perennial lakes |  | BLUE Stipple |
| Aeronautical data, except for Enroute and Area Charts — ICAO, where different colours may be required. Both contours may be used on the same sheet but, where only one colour is used, dark blue is preferred | Optional colours | MAGENTA DARK BLUE |

CHART SYMBOLS (Cont.)

GREEN

GOLDEN BUFF

WHITE

Optional colours

Areas which have not been surveyed for contour information or relief data are incomplete

Woods

| WHITE VIOLET |  | Tint for extreme elevations |  | SEPIA |  |
| --- | --- | --- | --- | --- | --- |
|  | ORANGE or BUFF | Tint for higher range elevations |  | BROWN |  |
|  | YELLOW | Tint for middle range elevations |  | BUFF |  |
|  | GREEN | Tint for lower range elevations | Optional colours | GREEN WHITE |  |
|  |  | Tint for areas below sea level | Optional colours | BLUE- GREEN LIGHT GREY |  |

HYPSOMETRIC TINTS

BLUE- GREEN

Note.— Basic tints are identical to those specified for the International Map of the World.